APRS.doc 6.8       AUTOMATIC PACKET REPORTING SYSTEM (APRS)

APRS is the result of my experience over the last 17 years with trying

to use packet radio for real-time communications or public service events.

Packet radio has great potential but so far has been best used for passing

large volumes of message traffic from point to point or into the national

distribution system.  It has been difficult to apply packet to real time

events where information has a very short life time.  Since APRS is shareware

everyone is encouraged to play with it all they want and to NOT feel compelled

to register.  I want APRS, or something like it, to become a defacto standard

for packet exchange of graphical information to help us all.

APRS avoids the complexity and limitations of a connected network.  It

permits any number of stations to exchange data just like voice users would

on a voice net.  Any station that has information to contribute simply sends

it, and all stations receive it and log it.  Secondly, APRS recognizes that

one of the greatest real-time needs at any special event or emergency is the

tracking of key assets.  Where is the Event Leader?  Where are the emergency

vehicles?  Whats the Weather at various points in the County?  Where are the

power lines down?  Where is the head of the parade?  Where are the VIP's?

Where is the mobile ATV camera?  Where is the hurricane?  WHERE IS THE DX???

Included on the distribution disk are several README text files on

applications of APRS such as for weather nets, direction finding, plotting

satellite contacts, and monitoring DX clusters.  You must read at least the

README.1st, DEMOS.txt, NewNotes.txt and HELP.txt files.  APRS uses UI frame

broadcasts and map displays.  There are three major display subsystems and

a number of other minor displays as follows:

LATEST BEACONS - This display maintains a list of the latest UI frame

received from each station.  In effect, this is a multi-station one-line

broadcast message system.  Since the lines contain the LATEST time of receipt,

this display shows if a station is still on line within the last few minutes.

In DX cluster mode, it accumulates a list of all users and what was their

latest command to the cluster.

POSITIONS - This display maintains a separate list of the positions of

each station often including a brief comment.  These posits show the latest

time of receipt and give an indication of the latency in the network over

unreliable paths such as HF.  They can also contain DF and or WX info.  In DX

mode, this list contains the posits of all DX spots and user-locations heard.

MAPS - Maps from .5 miles up to 2000 miles can be displayed. Stations

that are transmitting a properly formatted position beacon will be plotted.

Stations reporting a course and speed are dead-reckoned to their present

position.  A database of most National Weather Service stations is built in.

You can zoom in to any point on the globe!

MESSAGES -  In addition to the BEACON text one-line broadcasts, there is

an operator-to-operator message capability.  Any station can send one line

messages to any other station.  On receipt, the messages are acked and

displayed on the bottom of the receiving stations screen until the operator

hits the K key to kill them.

BULLETINS - This screen maintains a list of active all-net BULLETINS.

BULLETINS allow one station to send multiple lines to the BULLETIN page at

ALL stations.

TRAFFIC - This screen shows the last 23 lines of messages exchanged

by any stations on the net.  Is useful for "READING THE MAIL".  DX mode

TALK traffic also shows up here.

ALL PACKET LOG - This display is a time sequenced log of every new

beacon or one line message sent.  Beacons are logged the first time they are

received.  This is in contrast to the LATEST display which shows the most

recent time of receipt of a beacon text.  In DX mode, this screen shows a

chronological list of all DX/WX/WWV and ANNOUNCEMENTS heard.

HEARD LOG - This display maintains a count of the total number of

transmissions from each station per hour.  These statistics are ideal for

displaying the connectivity of the network over varying paths, such as HF, or

to see when stations enter and leave the net.

DIGIPEATER LIST - This display displays the raw packet header so that

APRS users can see what digipeater paths are being used by other stations.

The proper use of digipeaters is important in an APRS network.  An asterix

in the first column means that you hear the other station direct without a

digipeater!  Change your UNPROTO path to NONE to chat with those stations.

In DX mode, this list accumulates a list of all DX cluster messages

monitored.

STATION TRACKING.  Although APRS automatically tracks mobile packet stations

interfaced to GPS or LORAN navigation, it also tracks perfectly well with

manual reports or GridSquares.  APRS will use a GridSquare in brackets at the

beginning of a BText.  Additionally, any station can place an object on his

map including himself and within seconds that object appears on all other

station displays.  In the example of a parade, as each checkpoint with packet

comes on line, its position is instantly displayed to all in the net. Whenever

a station moves, he just updates his position on his map and that movement is

transmitted to all other stations.  To track other event assets, only one

packet operator needs to monitor voice traffic to hear where things are.  As

he maintains the positions and movements of all assets on his screen, all

other displays running APRS software display the same displays.  There is also

a Tracking command on the P display that will cause APRS to keep the map

display always centered on a selected object.

GRID SQUARES:  Because of the ambiguity of a grid-square position report, APRS

will not display a four 4 or 6 digit gridsquare report on map ranges less than

128 and 8 miles respectively.  Stations reported in the same grid square are

randomly offset to avoid cluttering of callsigns on top of each other in the

same grid square.  The P-list is annotated to indicate that the position is

approximate.  It is also very brief; six characters vice seventeen.  In GRID-

SQUARE mode, APRS will send your posit via GRID SQUARE format vice LAT/LONG.

Shortening the packet is an advantage when reporting via MIR, SAREX, or HF.

USING DUMB TERMINALS IN AN APRS NETWORK:  Even dumb terminal stations can

report their positions by taking coordinates off of a paper map and entering

their LAT/LONG into their beacon text.  Using the same map, he can plot the

location of other APRS stations as he sees monitors their posits.  If all

packet stations on all frequencies would include LAT/LONG or Grid Square in

their BTexts, APRS can be used as a general purpose network topology display

on ANY packet frequency! See the PROTOCOL.txt for details on APRS formats and

see FRQCOORD.txt for suggestions for using APRS as a freq-coord display tool.

SPACE APPLICATIONS:  APRS is a solution to the effective use of orbiting

packet digipeaters such as on the Shuttle, MIR, and AO-21.  The problem with

space packet is the saturation on the uplink which makes the use of a normal

CONNECTED protocol impractical.  A CONNECTED contact requires, a total of 5

successive and successful packets.  Not only does APRS reduce this to one

packet, but it also capitalizes on the most fascinating aspect of the amateur

radio hobby, and that is the display on a map of the location of those

stations.  If everyone simply inserted their LAT/LONG or Grid Square as the

first characters of their beacon text, everyone within the satellite footprint

would see the location of every successful uplink.  All it would take to

implement this capability is a single AMSAT news bulletin to ask all stations

to insert their POSITS in their beacon text.  No changes onboard the shuttle

or MIR would be required.  See SPACE.txt for further details.

FOX HUNTING OR DIRECTION FINDING:  APRS is an excellent tool for plotting the

location of a hidden transmitter, balloon, or interfering signal.  APRS will

display the intersection of bearing lines from a number of reporting stations

and also overlapping signal strength contours if only signal strengths are

reported.  Finally, APRS includes the Fade-Circle Search and Rescue technique

which can be used by a mobile with only an OMNI antenna to locate a hidden

transmitter.

To use APRS for DFing, each station having a bearing report or a

signal strength  on the target, simply enters that bearing using the OPS-DF

command.  His station will then not only report his location, but also a line

of bearing or signal strength contour.  All stations running APRS will see

any reported DF bearing lines on their maps.  Further, they can track any

DF vehicles using GPS or LORAN device on board.  There is an optional  Dopler

DF registration for direct connection to a Roanoke or Doppler Systems DF unit

for automatically plotting and transmitting instantaneous DF bearings.

Please note that APRS uses 360 degrees for North and 0 to indicate a signal

strength report.  For more DF info, see the DF.txt file.

WEATHER STATION REPORTING:  APRS position reports can also include the wind

speed and direction, as well as other important weather conditions.  APRS

supports a serial interface option to the ULTIMETER-II home weather station.

With this interface, your station includes WX conditions in your position

report for display at all other stations in the network.  All weather stations

show up as a bright blue circle, with a line indicating wind speed and

direction.  The NEXT command in APRS will successively highlight each WX

station in turn, so that all WX reports can be had at a glance.  See WX.txt

for more information.  APRS also has a database of the locations of most NWS

sites for instant display.  APRS can also crunch a file of NWS hourly WX

conditions and update all NWS stations on the map.  Finally, APRS users can

set WX alarms and be alerted when WX conditions exceed those values.

DX CLUSTERS:  The positional display and real-time user communications makes

APRS an ideal tool for the DX cluster user.  Not only does he get to see all

DX spots on the map, but by operating in the monitor only mode, he has reduced

the overall packet load on the DX cluster.  This is a benefit to everyone on

the channel.  Also the APRS monitroing station will see the SPOT as soon as

the first station gets it, rather than later on down the list.

FREQUENCY COORDINATION:  As more and more BBS's, DIGIS, and NODES begin to

include APRS position information in their routine BEACONS, APRS makes an

excellent tool for frequency coordination.  If the frequency coordinating

body makes available APRS backup files (or .HST files) showing the psoitions

of all coordinated services, both packet AND VOICE, then these files can be

distributed on BBS and by other means to all HAMS in the area.  By loading

these files, users can instantly see the established frequency users in a

geographical setting.  Add to this visual reference, the ability of APRS to

plot crude range ring contours based on antenna height and gain, and you can

see that APRS is a very useful tool for displaying the topology of any

network and also the interference ranges of voice repeaters...

PROTOCOL - Since the objective of APRS is the rapid dissemination of real-time

information using packet UI frames, a fundamental precept is that old

information is less important than new information.  All beacons, position

reports, messages and display graphics are redundantly transmitted but at a

longer and longer repition rate.  Each new beacon is transmitted immediately,

then 20 seconds later.  After every transmission, the period is doubled.  After

ten minutes only six packets have been transmitted.  After an hour this results

in only 3 more beacons; and only 3 more for the rest of the day!  All trans-

missions can be turned off using one of the CONTROLS commands.  But a transmis-

sion can be forced at any time by hitting the X key.  For details on the APRS raw packet

formats see the PROTOCOL.txt file.  The maximum period can be set to a period

even greater than the 15 minute default.  See the CAP section in AIRCRAFT.txt.

COMMANDS:   In most cases the keyboard is always active.  There is a mneumonic

relationship between all functions and the appropriate key.  For this reason,

the PC function keys are avoided.  (APRS processing of packets on the air is

continuous EXCEPT while waiting for the user response to a prompt.  These

prompts are surrounded with a blue box).  Commands fall in to 3 categories:

SCREENS:

Space Key           - Display map and all station locations

L - Latest beacons  - Displays the latest STATUS BEACON from each station

P - Positions       - Displays a list of all stations reporting positions

A - ALL packet log  - Keeps a chronological log of all beacons and messages

B - BULLETINS       - Keeps a list of all BULLETINS heard

R - Read Messages   - Displays the status of your in and out messages

T - Traffic         - Displays the last 23 lines of messages between stns

D - Digis Used      - Displays the digipeater paths being used by others

H - Heard Log       - Displays packets per hour per station for 24 hours

V - VIEW            - Displays all packets on a scrolling screen

SUB-MENUS:

F1- Help            - Select from a MENU of HELP commands

C - Controls        - Display a one line status of all control states

F - FILES Menu      - For Loading/Saving files, or Replaying tracks

I - Input commands  - Used to input posits, DF info or add OBJects to map

O - OPERATIONS      - Several commands for normal operations

M - MAP Functions   - Functions dealing with maps

W - Weather Menu    - Displays the number of beacons per hour per station

MESSAGES:

R - READ            - Displays your incomming and outgoing messages

S - Send            - Sends traffic to a station

E - Erase           - Erases outgoing traffic lines

K - Kill            - Kills incomming traffic lines

T - Traffic         - Displays message traffic between other stations

DEMONSTRATION FILE:  To see how the APRS system works on our frequency, use

FILES-LOAD to load the file called FREQ579.BK.  This file contains all the

local stations on 145.79 MHz in our area.  To see the tracking of the GPS

equiped Army/Navy game football run, load the file named FBALL.BK and replay

the file named FBALL.HST and select to see only FBALL, or CHASE1.  To see the

Marine Corps marathon event, load MARATHON.BK and replay the MARTHON.HST file.

See Details in README.1st.

HOOKING STATIONS:  The cursor can be moved to select any station on the map

using the arrow keys.  When near any station symbol, the ENTER key will

"hook" the station.  Detail information on that station will be displayed

on the bottom of the screen.  Alternatively, use the + and - keys to step

through each station one by one.  You may also use the cursor on the P or

L-list to hook a station or object.  Once hooked, several functions may be

performed:

1.  ALL BEACONS - The A key will list all beacons from that station.

2.  MOVE - Move the cursor to a new location and press the Insert key.

You are then prompted to enter in a new info as needed.

3.  DELETE - performed by hitting the D key.

4.  UPLINK - transmit the object to all other stations on the net

5.  QUIT - quit uplinking the object to the net.

6.  KILL - kill the object from all displays in the network

7.  ALARM - You will be alerted if that station ever moves.

8.  TRACK - APRS will always center display on selected station

9.  #MARK - Mark special stations so that only these stations will be

shown on the map when # is used instead of SPACE BAR for drawing a map.

If the * key is pressed, all symbols will be shown on the map, but only

the marked symbols (#) will show callsigns.

The hook function also works on the LATEST and POSITION display lists by

using the up/down arrow keys.  If a position exists, hitting the SHOW key

will display the map screen with that station centered on the display.

REPLAY:  The positions of any moving station can be replayed either from

memory or from a file.  Tracks are kept in on-line memory until 150 have been

saved, and then are saved to a HISTORY file.  During REPLAY, use the Calls

command to toggle on and off the display of callsigns, and use the HOME and

page keys to center and zoom the map display if the mobile station moves off

the screen.  During replay, use these commands:

C    - CALLsigns on/off

HOME - Homes map to presently displayed station

SPACE- Redraws the present map to remove track clutter

F    - Faster.  Speeds up playback

G    - Overlays the Civil Air Patrol Search and Rescue grids

M    - Medium replay speed

P    - Pause

S    - Slow.  Slows down playback

Q    - Quit playback.

PgUp/PgDn - Zoom in and out

FILES:  All APRS files are retained in five different sub-directories of

BAKS, LOGS, HSTS, SYSTEM and README.  There are several other files

used by the system:

MAPLIST.APR  - The directory of all map files.  APRS uses this file to find

out what map to use for a given cursor location.  Many other

MAPLIST.xxx files are now available.

BACKUP  .BK  - Automatic backup of system every time program is quit.  This

file is overwritten every time the program is quit.  It can

be reloaded by simply indicating the letter B for a filename.

MAPFIXer.BAS - The Qbasic program I use to fix, draw, and modify APRS maps!

SYSTEM (DIR) - Contains the following required files

IintTAPR.TNC - Setup parameters for your TNC (InitAEA.TNC for AEA)

RESTORE.TNC  - TNC commands used to restore your TNC after quitting APRS.

NWSDATA.DAT  - A sample file used to load National Weather Service data

NWSPOSNS.DAT - A file of the locations of all NWS sites

CAPGRIDS.DAT - A file of the Civil Air Patrol Sectional Aeronautical charts

DXCALLS.DAT  - Callsign prefix-to-LAT/LONG database for DX spots

XXXX.SYS     - Numerous brief files for APRS internal screens

CHESSBOARD:  To demonstrate the flexibility of APRS in reporting the movement

of objects on screens in a net, I have drawn a chessboard map in the center

of the Gulf of Mexico.  Any two stations can play chess easily using APRS by

placing pieces on the map using the INPUT-ADD command and updating their

positions using the cursor and INSert keys!  The full 32 pieces are already

loaded and saved in CHESS.BK.  To move a piece, just hook it, move the cursor

to the new position and hit the INSert key.  Other APRS stations will see the

game progress too!  You should consider going to an unused frequency so as not

to clutter an active APRS net, however.  Also, after a piece has been moved

and the new location seen by the other player, it is useful to Quit uplinking

it to minimize QRM.