FlexNet-Treiberdokumentation    DSK50.EXE         (C)1996 Tom Sailer HB9JNX

Stand: 16.1.96

Ident: DSK50

Treiber fr das Texas Instruments DSP Starters Kit TMS320C5x

Dieser Treiber ermoeglicht es, das DSP Starters Kit als Packet-Modem zu

verwenden und zwar entweder fuer 1200 Baud AFSK oder fuer 9600 Baud FSK,

kompatibel zum G3RUH-Standard. Das DSK wird an eine serielle Schnittstelle

angeschlossen. Es arbeitet als Modem und schickt einen Bitstrom zum PC, der

diesen auswerten muss (HDLC Protokoll decodieren), was Rechenleistung auf dem

PC kostet.

Als PTT-Leitung kann RTS der seriellen Schnittstelle verwendet werden

(ACHTUNG: Spannung von -12..12Volt!)

Hardwarevoraussetzungen: Ausser Rechenleistung und einer Standard-seriellen

Schnittstelle (16550 wird unterstuetzt) keine.

Parameter:

/?,/h    Beischreibung der Parameter

/c       COM Nummer

/b,/p    Basisaddresse der Schnittstelle (falls /c nicht verwendet wird)

/i       Interrupt-Nummer (falls /c verwendet wird, muss /i nachher kommen)

/a       Den AUXIN-Eingang benutzten

/d       DTR invertieren (wird benoetigt, um den DSP zu initialisieren)

/r       RTS invertieren

/s       Beschraenkt die Baudrate der seriellen Schnittstelle auf 12800, und

damit die Funkbaudrate auf 1200.

Das 9600 Baud-Modem ist sehr empfindlich auf DC-Pegel am Eingang. Man muss

also sicher gehen, dass das Empfaengersignal um die Masse zentriert ist!

Bei einigen RMNC-Karten ist das naemlich nicht der Fall.

Limitationen:

- Kalibration schaltet mit ca. 3s (bei 1200 Baud) Verzoegerung aus.

- Messung des TX-Delays anderer Stationen wird nicht unterstuetzt

Anmerkung:

Dank des schnelleren DSPs konnten einige Unschoenheiten des TMS320C26

DSK Treibers bei 9600 Baud FSK ausgeraeumt werden. So ist der Tx-Filter

wesentlich besser, und der Codec wird gerade noch innerhalb der Spezifikation

betrieben.

---------------------------------------------------------------------------

FlexNet driver documentation     DSK50.EXE        (C)1996 Tom Sailer HB9JNX

Date: 16.1.96

Ident: DSK50

Driver for use with Texas Instruments DSP Starters Kit TMS320C5x

This driver allows TI's DSK to be used either as 1200 baud AFSK modem or

as 9600 baud FSK modem, compatible to the G3RUH standard. The DSK works

as a modem frontend, sending a serial bitstream to the PC, which then

must decode it (HDLC protocol), which requires some processing power on the

host PC.

The RTS line of the serial port may be used as the PTT signal (ATTENTION:

Voltage level from -12 to 12V)

Minimal hardware configuration: A standard serial interface (16550 is

supported) and some processing power.

Parameter:

/?,/h    description of the parameters

/c       the COM port number

/b,/p    base address of the serial interface (if /c is not used)

/i       the IRQ number (if /c is used, /i must be placed afterwards)

/a       use the AUXIN input

/d       invert DTR (DTR serves as the reset line for the DSP)

/r       invert RTS

/s       limits the baudrate of the serial interface to 12800, and thus

the radio baudrate to 1200.

The 9600 baud modem is very sensitive to DC levels at the input. Make sure that

the receiver signal is centered around ground!

Limitations:

- Calibration finishes with about 3s delay (at 1200 Baud)

- Measurement of the TX delay of other stations is not supported

Note:

The faster DSP allowed the removal of some nasty limitations of the TMS320C26

DSK driver at 9600 baud FSK. The TX filter is now significantly better, and the

codec operates within its specification.

Autor:  Tom Sailer

Weinbergstrasse 76

CH-8408 Winterthur

Packet: HB9JNX @ HB9W.CHE.EU

EMail:  sailer@ips.id.ethz.ch