���     ��       �

�����  ���   ����  � ���

TST     ���  ��  �� ��   �� ��

��  ��  �����   �� ��

�����  ����  ����� ��� ���

��  ��

������  v 2.3  (C) Hugo Visser FL2JGI

������������������������������������������������������������������������������

����> Tstsign <����

������������������������������������������������������������������������������

� What is it?

�������������

Tstsign is a program, which can place standard texts above and under your

messages before a forward session in Tsthost. This way you can add your own

'logo' to your messages.

� What do you need for TSTSIGN...

���������������������������������

- a PC on which TSTHOST 1.4X can run.

- 42K (or more) free shell-memory under TSTHOST

- 100K of free disk-space

� The installation...

���������������������

Put TSTSIGN.EXE and TSTSIGN.CFG in your TSTHOST directory (usually

C:\TSTHOST). Adapt with a ASCII editor the TSTSIGN.CFG. A example of a

TSTSIGN.CFG:

#Cfg for TSTSIGN

$99999

# Your registration Number *

SUPCALL=FL2XXX NL2XXX

# Extra calls which may not signed.

NOSIGN=AUTO7P 7PSERV REQDIR

# Adresses which may not signed.

COPYMSG_OFF

# CopyRight text off.

KEEPLINE

# Keep standard-text from TSTHOST.

NICE_RE

# Adds spacing in Reply-msg's (just try and see)

ACK_P

# Add '/Ack' to P-messages.

Without TSTSIGN.CFG TSTSIGN will work, but with defaults. De standard-text

from TSTHOST will be erased. All other functions will not work. Only when

you have a registration-code this file is important (OR when you want to

keep the standard text from TstHost).

Make a <CALL>.HDR and a <CALL>.SIG file, which contains the Header-text and

the Signature-text for that <CALL>. Note: <CALL> is the call which need to be

signed.

Both files in ASCII and in the TSTHOST directory !!

Put TSTSIGN in your CRON.BAT to start TSTSIGN automatically. (else you've to

start it manually)

Replace (or add) de FTIME in your TSTHOST.CFG to 0 and add the option /F on

your command-line TSTSIGN. TSTSIGN will now forward automatically. This way

your mail will be forwarded when it is ready. Else TSTHOST starts a forward

session when you typing a message or some messages are not signed!

* When you use the Option SUPCALL, there must be seperated files, else

TSTSIGN will not add header or Signature-texts. The extra Standard TSTHOST

text will be removed wheter the file(s) exists or not.

* COPYMSG_OFF, NICE_RE, SUPCALL and ACK_P can only work on

a registered version.

Registration is free, adress at the end of this doc !

* Please note that with NICE_RE your messages could become a bit messy...

� Macro's

���������

It's possible to use macro's in your .HDR and .SIG files.

These macro's will be translated by TSTSIGN and replaced in your message.

You can use the following macro's:

%#           -  Number of the message (taken from the BID)

%B           -  The @BBS field of the message

%D           -  The date in numbers (06/01/1996)

%D1          -  The date in words (1 June 1996)

%D2          -  The day of the week.

%M           -  Callsign of the sender.

%O           -  Random reads a line from TSTSIGN.ORI and inserts it.

%R           -  Callsign of the recepient

%S           -  Insert Subject of the message.

%V           -  Version number of TSTSIGN.

� TSTSIGN.LNG

�������������

TSTSIGN.LNG is the language file for TSTSIGN, which is used for the date

macro's.

You can create your own language file with TS_LANG.EXE.

When you delete TSTSIGN.LNG, the default language will be English.

� Writing Messages...

���������������������

Start TSTHOST as usual. Write your messages. Every hour (if TSTSIGN is added

to your TSTHOST.CRN) TSTSIGN will sign your messages and wait 10 seconds or

'till a key is pressed. When you press a key (exept ESC), TSTSIGN will give

TSTHOST the command to Forward. Your messages will be forwarded with sign

automatically. When you press ESC, TSTHOST will be on your screen again and

your messages will not be forwarded this time.

When there will be no messages signed, but some messages exists which are not

forwarded, TSTSIGN will also wait 10 seconds and forward. When there are no

messages signed and no message unforwarded, TSTSIGN won't wait (registered

version).

You can add the text "/SKIP" in your message, TSTSIGN will skip this messages

when proccesing. This text must be at the beginning of a line ! (Left). When

you have KEEPLINE off, the first standard-line from tsthost will be removed

as well.

7+ Messages (status D) will not be signed!

� Use of TSTSIGN with a MACRO...

��������������������������������

You can start TSTSIGN with a macro. Then you can start it whenever you want

(and forward). You must add the following text to your TSTHOST.MAC to enable

this function:

:TSTSIGN

10 RU C:\TSTHOST\TSTSIGN.EXE /F

This way you can Sign (and forward?!) by typing "TSTSIGN" at the TSTHOST

CMD> mode.

� Registration...

�����������������

It would be fine when you register your copy of TSTSIGN by the author of this

program. This way he knows who uses the program. (And if it's effortless

to distribute). It's completly free. This is what you've got to do:

Write a message to one of the following adresses:

Ax.25 Packet radio (27 MC) : FL2JGI @ FL3FLE.FL.NLD.EU of

Via Internet (4ALL gateway): NL2JGI@CBPACKHV.XS4ALL.NL

State your call in your message which you use for TSTHOST mail (AX MY in your

TSTHOST.CFG) and you'll get a message back with your code!

IMPORTANT!: Your code cannot be used for other MYCALLS, only for the mycall

it's calculated for. This is not applicable to SUPCALLs.

When you cannot send a message direct to me, because you don't have internet,

or you operating on a different frequency-band with packet, ask someone else

to send your message.

� Responsability...

�������������������

This program is manufactured by Hugo the FL2JGI with Turbo Pascal 6.0 (TM)

for own use. He will not take any responsability for damage by using this

program.

The program may not be sold without permission!

The program is fully tested, but there can be a bug somewhere. When you

discover one, the author will be gratefull when you send him a message, so

he can solve the cause.

The documentation is writed by Ren� de NL2RDS with Word Perfect 5.1+ (TM).

� At last, but not least...

���������������������������

I (FL2JGI) want to thank the following persons:

- Ren� NL2RDS for the idea of TSTSIGN and (many) testing. Without him the

program was not get this far!. I'm also want to thank him for writing the

documentation.

- Chris NL2CHR for the assembly code 8-)